WATER COOLER MODELS

POUPQ1PK, POUPQ1PHS

# INSTRUCTIONS

A.	INSPECTION

Inspect the carton and water cooler for evidence of concealed damage. Damage claims should be filed with the carrier.

B.		MAINTENANCE

- Periodically remove dirt and lint from the condenser. Inspection should be done every three months.  If cleaning is required, disconnect the power supply cord, then clean the condenser with a small stiff non-wire brush.  Following this procedure will ensure adequate air circulation through the condenser for efficient and economical operation.

  - NOTE: Under no circumstances should the cabinetry be cleaned using a water jet

- A cone shaped strainer can be found in the tube leading into the mechanical float valve on the reservoir cap.  It can be removed and flushed clean.  Be sure to replace it in the tube once cleaned.

C.	OVERLOAD PROTECTION

The compressor motor is equipped with an automatic reset protector which will disconnect the motor from the line in case of an overload.

#### HOT TANK THERMOSTATS

ON HOT MODELS, THE HOT TANK THERMOSTATS ARE NOT DESIGNED TO OPERATE WET. AN ELECTRICAL FAULT MAY OCCUR IF THE THERMOSTATS ARE WETTED.

- Do not steam or wet clean in the thermostat area.

- All minor water spills must be wiped up immediately after loading or unloading a bottle onto or from the cooler.

- Where major water spills or leaks have occurred (internal components have got wet), do not operate the hot tank until the thermostats have been given sufficient time to dry out (48 hours), or the thermostat(s) have been replaced.

E.	LUBRICATION

This unit is equipped with a sealed compressor that requires no additional lubrication.

F.	TO PUT WATER COOLER INTO SERVICE

- This unit is suitable for indoor use in domestic, commercial & industrial locations. They should not be sited in locations where they may be subjected to rain or snow.

- The water cooler should be located a minimum of 2" from the wall to assure adequate condenser air circulation.

- Power cords fitted to the cooler must be tied securely to the back of the unit (base) using the cord restrainer supplied, in such a way that it is not possible to remove the power cord from the appliance by pulling on the flexible cord.

- Ensure that the appliance is positioned to give access to the plug after the unit is installed.

- If the supply cord is damaged, it must be replaced.

- Recommended ambient temperatures for correct operation of the unit are between 10ºC and 32ºC

- For safe operation, the unit should always be placed on a level surface.

- Cold water temperatures may be adjusted via a slotted screw on the body of the control (can be accessed through wires on the condenser). Turning this screw in the clockwise direction will make the water colder (and vice versa)

- The machine should only be connected to a potable drinking water supply.

- The water pressure in water supple should be not less than 0,6MPa

- The A-weighted sound pressure level of this appliance is below 70dB

##### CAUTION: DO NOT INSTALL THE WATER COOLER IN AN AREA WHERE WATER JETS MAY BE USED

- Check the available power supply against the water cooler data plate to ensure correct electrical service (DO NOT PLUG IN AT THIS TIME)

- If filter(s) are to be added, follow filter manufacturer’s recommended instructions.

- Open the access door at the rear of the unit by removing the two screws

- Connect the mains water supply to the shut off valve using  ¼” tubing   (Note: 6mm tubing should NOT be used)

WARNING:  A pressure regulator, such as a Slow Flow regulator, must be installed in front of the unit’s water inlet if the water pressure (including any possible pressure spikes) could exceed 100psig.

- Ensure shut off valve is in opened position

- Close the access door and replace the two screws

- Reservoir will now begin to fill

Cook and Cold Model

- Draw water from the faucets

- For models with an IEC power inlet,

A) Connect the female portion of the approved power cord into the IEC inlet.

B)  Insert the plug top of the power cord into the electrical supply socket.

- Allow about 30 minutes for the water to chill after the cooler is plugged in.  (Optimum cold water temperatures will not be reached for several hours).

##### Hot and Cold Model

- Open the hot faucet until water flows freely

- For models with an IEC power inlet,

A) Connect the female portion of the approved power cord into the IEC inlet.

B)  Insert the plug top of the power cord into the electrical supply socket.

- Place the hot tank heater switch in the “ON” position

- Allow the cooler to operate for about 30 minutes for chilled and hot water dispensing  (Optimum cold water temperatures will not be reached for several hours).

CAUTION  :  ON MODELS WITH HOT TANKS, FAILURE TO FILL THE HOT TANK WITH WATER BEFORE TURNING ON THE HOT TANK HEATER SWITCH CAN CAUSE PHYSICAL DAMAGE TO THE UNIT.

CAUTION  :  WATER FROM THE HOT FAUCET CAN SCALD

G.		SAFETY FLOAT

The cooler is equipped with a safety float to prevent overfilling in case the fill float fails.  If the fill float should fail, the safety float will be activated as the water level rises.  When the safety float is activated, water flow to the reservoir will be shut off.  Press the blue reset button about half way down on the reservoir cap until you hear the water flow restart, then call your service representative.  CAUTION:  A hard impact or jarring of the unit while it is filling may trip the safety float.

H.	TO DISCONTINUE USE OF WATER COOLER

Disconnect the power supply cord from the power supply.

Cook and Cold Model

- Turn off the mains water supply

- Drain water in reservoir through faucets.

- ALWAYS DRAIN ALL WATER WHEN FREEZING TEMPERATURES ARE ANTICIPATED AND BEFORE SHIPPING THE WATER COOLER.

Hot and Cold Model

- Place the hot tank switch in the “OFF” position.

- Draw water from the hot faucet until the water is cool.

- Turn off the mains water supply.

- Drain water in the reservoir through the faucets.

- Place a container under the end of the drain tube – the drain has a white plastic screw-on cap that exits the cooler through the condenser.

- Remove the screw-on cap to drain the tank.

- Refit the screw-on cap when the tank is empty.

CAUTION : DO NOT DRAIN THE HOT TANK WITHOUT DRAWING WATER FROM THE HOT FAUCET UNTIL THE WATER IS COOL - HOT WATER CAN SCALD

CAUTION : ALWAYS DRAIN ALL WATER WHEN FREEZING TEMPERATURES ARE ANTICIPATED AND BEFORE SHIPPING THE WATER COOLER.

I.	ALTERNATE FAUCETS

A safer faucet that deters accidental dispensing of water by toddlers is available. This new faucet is simple and easy to use with one hand, yet it adds a new measure of protection to your water cooler. See your dealer for details.

#### SANITISING INSTRUCTIONS

If sanitising is required, a solution can be made using sodium hypochlorite solution at a maximum concentration of 200 parts per million (add 2 ml of 10% sodium hypochlorite to 1 litre of water). DO NOT USE A STRONGER SANITISING SOLUTION. Wash the cooling tank & baffle in the solution. Rinse immediately. Do not allow sanitising solution to remain in the cooling tank for more than 5 minutes. On hot models, do NOT allow the sanitising solution to enter the hot tank; plug the hot tank inlet. Dispense solution through cold faucet and cook faucet (if supplied). Rinse through faucet(s) with clean water. Remove plug from hot tank inlet. Drain hot tank and flush with clean water.

###### CLEANING

The following cleaners are acceptable for use on the cooler’s plastic cabinetry and plastic drip tray receptor

Mild soap and water	Fairy®

Do NOT use bleach or any cleaning agents containing bleach or chlorine

L.   	REFRIGERANT

This product contains the hydrofluorocarbon refrigerant R-134a which has a Global Warming Potential of 1,300

WARNING

THIS APPLIANCE MUST BE EARTHED.

This appliance is not suitable for unsupervised use by young children or aged and infirm persons.

The warranty, Underwriter’s Laboratory certification and CE certification for this machine are automatically void if any alteration, modification, or combination with any other machine or device is deemed to be the source of any claim. The UL and CE certification may be void as the result of any alteration or modification.

The manufacturer accepts no liability resulting from any alteration, modification, or combination with any other machine or device.

EXCEPT AS SET FORTH HEREIN, THE MANUFACTURER MAKES NO OTHER WARRANTY, GUARANTEE, OR AGREEMENT EXPRESSED, IMPLIED OR STATUTORY, INCLUDING ANY IMPLIED WARRANTY OR MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE

OASIS East Sp.z o.o.

Gutenberga 20 str

44-164 Gliwice SEZ, Poland

Tel:   +48 32 332 6501   ;   Fax:  +48 32 332 6500

www.oasis.ie

030099-297 Rev A								     ©Copyright 2004 OASIS Corporation