|  | # Operating Instructions for B1DR, B1AR Watercoolers |
| --- | --- |

A.   INSPECTION

Inspect the carton and water cooler for evidence of concealed damage. Damage claims should be filed with the carrier

B.   MAINTENANCE

- Periodically remove dirt and lint from the condenser. Inspection should be done every three months.  If cleaning is required, disconnect the power supply cord, then clean the condenser with a small stiff non-wire brush.  Following this procedure will ensure adequate air circulation through the condenser for efficient and economical operation.

  - NOTE: Under no circumstances should the cabinetry be cleaned using a water jet

- FreshGuard helps prevent foreign matter from entering the cooling tank. Its parts should be inspected, cleaned and replaced (if necessary) at regular intervals. Operation of the cooler without FreshGuard components in place may allow airborne dust particles to enter the cooling tank.

- Cleaning of water contact components

  - The European Bottled Water Association recommends that water contact parts should be cleaned and sanitised every 3 months. It is recommended that this process is carried out by trained personnel

C.   OVERLOAD PROTECTION

    - The compressor is equipped with an automatic reset protector that disconnects the motor from the line in case of an overload.

D.   LUBRICATION

    - This unit is equipped with a sealed compressor that requires no additional lubrication.

E.   TO PUT WATER COOLER INTO SERVICE

      - This unit is suitable for indoor use in domestic, commercial & industrial locations. They should not be sited in locations where they may be subjected to rain or snow.

      - The unit should be operated with the stability cord at the rear of the unit fixed to the wall at all times.

      - The stability cord wall fixing should be mounted securely to the wall at a height of 660mm from the floor. The unit should be located approximately 140mm from the wall. Ensure that the cord is taut.

      - Power cords fitted to the cooler must be tied securely to the back of the unit (base) using the cord restrainer supplied, in such a way that it is not possible to remove the power cord from the appliance by pulling on the flexible cord.

      - Bottles should be no larger than 19 litres

      - When fitted to a cooler the height from the floor to the top of the bottle should not exceed 1.4 metres.

      - Ensure that the appliance is positioned to give access to the plug after the unit is installed.

      - If the supply cord is damaged, it must be replaced.  A replacement cord of type X attachment is available from OASIS

      - Recommended ambient temperatures for correct operation of the unit are between 10ºC and 32ºC

      - For safe operation, the unit should always be placed on a level surface.

      - Check the available power supply against the water cooler data plate to ensure correct electrical service. (DO NOT PLUG IN AT THIS TIME )

      - Cold water temperatures may be adjusted via a slotted screw on the body of the control (can be accessed through wires on the condenser). Turning this screw in the clockwise direction will make the water colder (and vice versa)

##### CAUTION: DO NOT INSTALL THE WATER COOLER IN AN AREA WHERE WATER JETS MAY BE USED

Cook and Cold Model

      - Invert a water bottle on top of the cooler.

      - Draw water from the faucets

      - For models with an IEC power inlet,

  - Connect the female portion of the approved power cord into the IEC inlet.

  - Insert the plug top of the power cord into the electrical supply socket.

- Allow about 30 minutes for the water to chill after the cooler is plugged in.  (Optimum cold water temperatures will not be reached for several hours).

Hot and Cold Model

- Partially fill the reservoir - about 3/4 full

- Open the hot faucet until water flows freely

- Invert a water bottle on top of the cooler.

- For models with an IEC power inlet,

  - Connect the female portion of the approved power cord into the  IEC inlet.

  - Insert the plug top of the power cord into the electrical supply socket.

- Place the hot tank heater switch in the “ON” position

- Allow the cooler to operate for about 30 minutes for chilled and hot water dispensing

CAUTION  :  ON MODELS WITH HOT TANKS, FAILURE TO FILL THE HOT TANK WITH WATER BEFORE TURNING ON THE HOT TANK HEATER SWITCH CAN CAUSE PHYSICAL DAMAGE TO THE UNIT. WATER FROM THE HOT FAUCET CAN SCALD

F.   TO DISCONTINUE USE OF WATER COOLER

        - Disconnect the power supply cord from the power supply.

Cook and Cold Model

- Remove bottle from reservoir.

- Drain water in reservoir through faucets.

- ALWAYS DRAIN ALL WATER WHEN FREEZING TEMPERATURES ARE ANTICIPATED AND BEFORE SHIPPING THE WATER COOLER.

Hot and Cold Model

- Place the hot tank switch in the “OFF” position.

- Draw water from the hot faucet until the water is cool.

- Remove the bottle from the reservoir.

- Drain water in the reservoir through the faucets.

- Place a container under the drain valve

- Turn the drain valve counter clockwise to open

- Turn the drain valve to the closed position when the tank is empty

- ALWAYS DRAIN ALL WATER WHEN FREEZING TEMPERATURES ARE ANTICIPATED AND BEFORE SHIPPING THE WATER COOLER.

G.   REMOVABLE RESERVOIR

  - This cooler is designed with a removable reservoir. The reservoir can be replaced with a sanitised unit during routine maintenance services.

To Remove Reservoir

    - Drain the cooler as described in section F.

    - Pour hot water over the reservoir sleeve to aid in removing the reservoir by melting any ice on the sleeve.

    - Drain any additional water from the reservoir through the cold faucet.

    - Remove the faucet cover as follows:

          - Press down the top of the cover, causing the top mounting lugs to spring open

          - Lift the cover over the faucet levers

          - Unhook the cover from the two lower mounting holes

    - Unscrew the two faucets from the reservoir by turning the faucets counter clockwise.

    - Grasp the top of the reservoir around the opening and pull upward in a straight motion.

      - Note: Take care when removing the reservoir that any water remaining in the bottom of the reservoir, does not fall into the appliance

Installing New Reservoir

        - Align the reservoir with the probe.  Place one hand inside the reservoir on the sleeve and push downward on the reservoir until it is resting on the insulation.

        - Reattach the faucets to the reservoir by turning them clockwise, ensuring that

          - They are on the correct thread with the reservoir.

          - The blue faucet is on the right hand side of the cooler

          - Both faucets are oriented so that their outlets align with the holes in the cover

        - Reattach faucet cover as follows:

          - Insert two lower lugs into holes in the cabinet below faucets

          - Push cover over faucets

          - Push top lugs into mounting holes

          - Push cover up into position

H.   ALTERNATE FAUCETS

            - A safer faucet that deters accidental dispensing of water by toddlers is available.  This new faucet is simple and easy to use with one hand, yet it adds a new measure of protection to your water cooler. See your dealer for details.

J.   COOLER TRANSPORTATION

            - Whenever a cooler is being transported between locations, it should be kept in the upright position. If it is necessary to transport the cooler with it positioned on its side, then the cooler should be stored in the upright position for 24 hours before being switched on. This will prevent any damage to the compressor.

            - THE WATERCOOLER MUST NOT BE LIFTED BY THE FAUCETS – THIS WILL DAMAGE THE RESERVOIR. A CARRYING HANDLE IS PROVIDED ON THE REAR OF THE UNIT.

K.   REFRIGERANT

This product contains the hydrofluorocarbon refrigerant R-134a which has a Global Warming Potential of 1,300

### WARNING

- THIS APPLIANCE MUST BE EARTHED.

- This appliance is not suitable for unsupervised use by young children or aged and infirm persons. The warranty and CE certification for this machine are automatically void if any alteration, modification, or combination with any other machine or device is deemed to be the source of any claim.  The CE certification may be void as the result of any alteration or modification.

- The manufacturer accepts no liability resulting from any alteration, modification, or combination with any other machine or device.

- EXCEPT AS SET FORTH HEREIN, THE MANUFACTURER MAKES NO OTHER WARRANTY, GUARANTEE, OR AGREEMENT EXPRESSED, IMPLIED OR STATUTORY, INCLUDING ANY IMPLIED WARRANTY OR MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

030099-303 Rev B								     ©Copyright 2009 OASIS Europe