Dispensing Water

- Depending on the model of cooler, it will have two, three of four push buttons located on the front panel as shown below.

- Water will continue to be dispensed while you keep the relevant button depressed.

- Water is dispensed as follows:

  - White button:	 – ambient water

  - Blue button: – cold water

  - Red buttons: - hot water

- Note: As a safety feature, you must press the two red buttons at the same time to dispense hot water

Cup Dispenser Unit

(P1PVCD models only)

- The unit contains a spring-loaded cup dispenser that is built into the lid. The cup dispenser is capable of storing approximately 70 cups.

- To fill the cup dispenser, take a stack of cups and turn them up-side down. Place the cup stack on top of the rubber ring in the cup dispenser & press down firmly.

- To take a cup from the dispenser, then hold the cup lightly and pull it out of the dispenser

Emptying the Drip Tray

- The drip tray comes with an optional self-drain facility. Check with your service provider if this option has been installed – if you have a self-draining drip tray then you do not need to remove it for emptying.

- To remove the drip tray, you need to lift it up by about 30mm before you pull it forward.

- The easiest way to lift out the drip tray is to place your thumb on the front face of the drip tray, and then hook your forefinger through the hole in the centre of the grille

Cleaning the Cooler

- Before cleaning the cooler, you should ensure that the unit is unplugged from the power supply

- Under no circumstances use abrasive cleaning products or chlorine based cleaners – they will damage the surface finish of the coolers.

- Use mild soap to clean the panels

- OASIS recommend that you use an anti-bacterial wipe to clean the water outlet on a weekly basis